The removal of telangiectasia

with JETT PLASMA LIFT MEDICAL

Version

Teleangiectasia 2019/01 ENG

It should be first evaluated, whether it is really telangiectasia on the skin (i.e. a localised accumulation of dilated blood vessels, capillaries or venules) and not any inflammatory change of skin colour or another skin disorder.

Check, whether they are the malign or pre-malign conditions of lesions or not. It is important to focus on a differential diagnosis of basaloma, squamous cell carcinoma and melanoma.

## THE PRE-TREATMENT PROCEDURE

- The information on contraindications

- Before the treatment, you should be informed about possible contraindications that could exclude the treatment (pacemaker, Holter monitor, another implanted electrical device, epilepsy, pregnancy, metal implants in the treated area).

- The examination of a lesion

- Evaluate the character of affection;

- Check the skin condition;

- Then, include the patients with telangiectasia into the indicated conditions.

- The disinfection and the application of local anaesthesia

- Remove the make-up;

- Disinfect thoroughly the skin (with regard to the fact that JETT PLASMA LIFT MEDICAL works with electric energy, we recommend to use an alcohol-free disinfectant);

- Anaesthesia application:

    - In case of small lesions, apply anaesthesia in the form of the anaesthetic cream with a minimum content of 5% Lidocaine or Tetracaine (e.g.: Emla cream or Lidocaine gel);

    - Wait for the cream effect for minimum 20 minutes;

    - It is suitable to use occlusion to speed up the effect of anaesthesia (cover the applied cream with an airtight foil – plastic foil);

    - In case of larger lesions, apply the local anaesthesia in the form of an injection (Supracaine 4%, Mesocaine 1%, Lidocaine, etc.).

## THE TREATMENT PROCEDURE

- The setting of JETT PLASMA LIFT MEDICAL

- Select a golden pointed head for the treatment and fit it on the device;

- Plug in the device. The message “TAPE” will light up on the display;

- Connect a blue cable to the electrode and stick it on the patient´s skin – the back side of the upper arm is suitable where the possible irritation of skin is much smaller thanks to a thicker skin;

- According to safety rules, connect the blue cable to the device only now to avoid any contact of the fitted head with the patient´s body and unprotected parts of your body;

- Having connected both cables and having stuck the electrode, the message “-0-“ will appear.

- The preparation of a patient

- Check the position of the body part with a lesion and the lighting which should be most comfortable both for the patient and the doctor;

- Disinfect again the skin using an alcohol-free disinfectant;

- Wait roughly 1 minute prior to start the treatment until the disinfection is evaporated.

- Treatment

- Put on a pair of sterile gloves

- Set the intensity 6 – 8 on the device (the selection of an intensity degree depends on your subjective consideration. Select 8 for better results and 6 or 7 for a more sensitive patient);

- Get closer to the patient´s skin with the head tip and as soon as you get to 4 mm above the skin surface, the spark discharge will ignite;

- If the tip of the head remains in a correct height above the skin, the plasma discharge will be continuous;

- Remove the telangiectasia by applying the discharge point by point along the whole length of a vessel – the points should be approximately 3 mm from each other.

- After the treatment, disinfect thoroughly the whole treated area.

- Disconnect the blue cable from the device and unstick the electrode from the patient´s skin.

## AFTER-TREATMENT THERAPY

- After the treatment, clean the skin with a disinfectant and cover it by a plaster as the case may be. (Keep the plaster maximum until the evening). By no means may you apply common corrective cosmetics.

- The skin can be washed up in the evening after the removal;

- Disinfect the treated places two times a day for 2 – 3 days after the treatment;

- After disinfection, it is suitable to apply a special regeneration gel containing hyaluronic acid above 1 % intended for an injured skin twice a day for the next 5 – 7 days

- After the regeneration gel dries up (approx. 10 min.), corrective cosmetics (make-up) can be applied on the skin;

- Protect the skin from solar radiation by applying the creams with SPF 50+ and sun glasses and head cover for a minimum of 7-10 days after the treatment;

- We recommend seeing the doctor after a week.

## RESULTS

The skin appearance after the treatment:

| 2 – 3 days after the treatment | Small scabs (seldom erythema, oedema) |
| --- | --- |
| 4 days after the treatment | Small scabs start to peel off (without any erythema, without any oedema) |
| 5 – 8 days after the treatment | Now, the skin is without any small scabs and without any colour changes. Make-up can be applied in this phase. Rejuvenation is visible. To support healing, we recommend to apply LiftVital serum two times a day for the period of 14 days |

If the treatment was performed correctly according to the instructions, the vessel should be removed immediately. Or perhaps a carbonised layer may remain on the skin – a small scab, which will sequestrate within several days.

Healing takes usually one week. In extraordinary cases, it may take a longer time. If a larger lesion was removed, its healing may last up to 14 days.

## FOR MORE INFORMATION AND TO SEE THE ILLUSTRATIVE VIDEOS, PLEASE VISIT OUR YOUTUBE CHANNEL

|  | www.youtube.com/jettplasmadevices |
| --- | --- |